Procedure to install the KODAK SL10 OR SL25 Distance camera on Prodigi Connect 12

Please take note that for the Kodak camera installation, no app is required to use within Prodigi software. Prodigi will be compatible with both models of cameras, but allow pairing of one at a time only.  Prodigi Connect 12 version 1.5.1 is mandatory to use Kodak cameras.

Pairing wireless of the camera to the Tablet

- Perform initial recharge of camera.

- Power the Kodak camera On (press power button and lens will deploy).

- Tablet: Go to your Android desktop and tap on Settings.

- Tap on the “Connections” tab.

- Tap on Wi-Fi.

- Tap on the Kodak PIXPRO-SL10_XXXX “network” (does not require Internet).

  - If SL25 model, choose PIXPRO-SL25_XXXX

  - XXXX are 4 unique characters to distinguish same make cameras, but unique channel to pair or use.

- Enter the pairing code: 12345678 and Connect button to pair.

It is recommended to reboot the Tablet. For this, press and hold the Power button of Tablet; and select “Restart”. Follow instructions.

Notice 1: If a Sony QX camera was installed on a Prodigi Tablet, Sony play memories app must be uninstalled before pairing of Kodak SL camera.

Notice 2: Only one camera can be used at once, therefore, many may be paired. Power On camera required for session, and set second camera Off.

Notice 3: HumanWare has a special software for the Kodak cameras.  Off-the-shelf cameras will not work with Prodigi.

Notice 4: If you require video functionality, you must install Kodak PixPro application and insert a uSD card to save file.  Prodigi has no video capability.